2.2  GEOLOGY

Comprehensive geological evaluations of the Skyline permit area, performed by and for the Permittee, have established the baseline environmental data necessary for mine development.  Future studies will be necessary to refine present concepts, monitor the existing geological/hydrological environment and provide additional data to be used for the continual mine planning process.  Continuation of this work will maximize the recovery of coal reserves by allowing increased accuracy in predicting coal seam discontinuities and will optimize safety by early identification of geology-related potential mine hazards.

2.2.1  Stratigraphy

Rocks mapped on the Skyline permit area (Plate 2.2-1) and adjacent areas (Plate 2.2.1-2) are of Upper Cretaceous age.  Also shown on Drawing 2.2.1-1 are faults which have been encountered either during the mining process or as a part of exploration activities.  The oldest stratigraphic unit, the Star Point Sandstone, underlies and intertongues with the lower portion of the coal-bearing Blackhawk Formation.  Overlying the Blackhawk is the Castlegate Sandstone, the basal member of the Price River Formation, and the youngest unit exposed in the area.  Figure 2.2-A shows the generalized stratigraphic sequence.

2.2.2  Star Point Sandstone

The Star Point Sandstone is a prominent cliff-former and an important marker horizon throughout the region.  The unit is the basal formation of the Mesaverde Group in the Wasatch Plateau, which overlies the Mancos Shale.

Only the upper part of the Star Point Sandstone is exposed in the area as it intertongues with the lower Blackhawk Formation.   The upper most tongue of Star Point outcrops in the Skyline Mine 3 portal area.  Major outcrops occur east of the Connelville Fault in the South Fork of Eccles Canyon and about 1,800 feet east of the Manti LaSal National Forest boundary in Eccles Canyon.

INCORPORATED

12/02/02

Division of Oil, Gas & Mining

Revised: 11/04/02	2-5

The upper part of the Star Point Sandstone is mostly a fine to medium-grained, sub-rounded to well rounded, light-colored sandstone, with a "salt and pepper" appearance.  Approximately the top 20 feet tend to have a pronounced white appearance in outcrop.  The massive beds exposed in the surrounding Skyline area are moderately well consolidated.

2.2.3  Blackhawk Formation

The Blackhawk Formation is the coal-bearing portion of the Mesaverde Group in the Skyline area.  The lower formational contact with individual tongues of the Star Point Sandstone is generally sharp.  Only on the high ridges in the northwest corner of the permit area is the upper formational contact with the overlying Castlegate Sandstone present.  The Blackhawk formation is atypically thick in the Skyline area and ranges from 1,700 to 1,900 feet in thickness where the complete section is present.

Due to its lenticular bedded nature, no persistent marker horizon occurs within the Blackhawk Formation.  The lower coal-bearing unit of the Blackhawk consists of 100 feet of fine-grained sandstone and siltstone, with zones of dark, carbonaceous shale and siltstone.  The uppermost carbonaceous zone in this unit contains, in places, one or more thin lenticular coal beds.  The thickest of these beds is usually referred to as the Flat Canyon seam, only a portion of which is considered mineable.

Three mineable coal seams are present in the basal coal zone of the upper coal-bearing unit of the Blackhawk formation.  In ascending order, they are the Lower O'Connor "A", the Lower O'Connor "B", and the Upper O'Connor seams.  These coals are of high-volatile B rank and in general contain few partings and little pyrite.  These coal seams are attrital, with midlustrous attrital being the most common coal lithotype.  Some resin is observed in all three seams.

The upper coal zone of the upper coal-bearing unit of the Blackhawk Formation generally contains carbonaceous shales and thin coal beds.  This zone occurs from 480 to 590 feet above the Storrs Sandstone tongue of the Star Point.  These coals are discontinuous with only local development of mineable thicknesses.  In the southeast corner of the leasehold area, a coal bed known as the

INCORPORATED

12/02/02

Division of Oil, Gas & Mining

Revised: 03/04/93	2-8

McKinnon seam appears to be of mineable thickness over an area of approximately 1,100 acres.  Similar to the lower mineable coals, the McKinnon seam is of high-volatile B rank.  This seam differs from the lower mineable coals in that partings are common and lateral thickness changes are pronounced.  In addition, the McKinnon seam is often very rich in megascopic resin.

The Blackhawk Formation consist mainly of lenticular sandstone, siltstone and claystone deposits.  Because of the lenticular nature of bedding in the formation, it is difficult to correlate individual horizons (with the exception of the major coal seams).  The sandstones are thin to thick bedded, ranging in grain size from very fine to coarse.  Argillaceous rocks include claystone, clay shale, silty shale and siltstone.  Carbonaceous and coaly rocks are present in every gradation ranging from slightly carbonaceous shale to coal.

2.2.4  Intrusive Igneous Rocks

Igneous dikes classified as "lamprophyre" cut the sedimentary rocks in the area.  The age of the intrusive rocks is in the 20 to 30 million year range and no dikes have been observed to cut Tertiary rocks in the area.  Where dikes cut the coal seams, a band of from one to five feet of metamorphosed coal is found adjacent to the dike.  The dominant trend of the dike swarm is nearly east/west.

2.2.5  Surficial Deposits

Unconsolidated soil, gravel, alluvium, landslide deposits, etc., mask many of the geological features of the Skyline permit area.  A description of these deposits is included in the Geotechnical section.

2.2.6  Structural Geology

The Clear Creek anticline is a major structural feature of the northern Wasatch Plateau.  In the Skyline area, large faults of the Pleasant Valley fault zone form a north-south oriented graben along the axis of the anticline.  The permit area is situated on the western limb of the anticline.  Strata dip to the northwest at the north end of the permit area, almost west at Eccles Canyon, and southwest at the south end of the permit area.  Dips range from three to six degrees on the permit area.

INCORPORATED

12/02/02

Division of Oil, Gas & Mining

Revised: 03/04/93	2-9

Major faults are nearly vertical, trending north-south to northeast-southwest in the area.  The largest of these, the Connvelville Fault, forms much of the eastern boundary of the permit area.  Within the fault block east of the Connelville Fault, the Belina Mine has encountered very small displacement, east-west faults which deviate from vertical with hades of up to 30 degrees.  Of the three major fracture/fault features known to have displacement, only the Connelville Fault is sufficiently developed to displace structural contours.  The faults, known locally as the Valentine Fault and the North Joes Valley Fault, are thought to have considerably less displacement than the Connelville Fault. Both of these faults are located outside the permit area to the south and southeast.  The Valentine Fault trends northeast and is located east of the Electric Lake Dam in Valentine Gulch.  The North Joes Valley Fault dies out south of Electric Lake.  The Connelville  gradually disappears north of the permit area.  The Connelville and Valentine Faults are vertically displaced downward to the west, and the North Joes Valley fault is vertically displaced downward to the east.

The O’Connor Fault is located approximately one to 1.5 miles east of and roughly parallel to the Connelville Fault and is offset down to the west.  Both the O’Connor and Connelville Faults slowly die out to the south of the permit area.  The Connelville Fault enters the Electric Lake basin north of Coal Canyon and the O’Connor Fault enters the Electric Lake basin south of Cox Canyon, just northwest of the dam.

The Connelville Fault is a complex fault zone with a width of up to 1,000 feet.  The cumulative displacement across the zone appears to increase from approximately 55 feet in the Winter Quarters Mine north of Skyline to 250 feet or more near the southern edge of the Skyline permit area.  Individual faults within the zone have much smaller, and highly variable displacement.  The Connelville Fault, as shown in Plate 2.2-1, is near the western edge of a complex zone, although segments of the zone may be encountered west of the map location shown.

Four major jointing and fracture orientations have been mapped on the leasehold.  The most common orientation observed within the coalbeds and immediate roof and floor strata are a set of joints spaced approximately 1 to 3 feet apart with a N80oW orientation.  This joint set is only occasionally observed in surface outcroppings.  A second joint orientation observed in the mines as well as in surface outcroppings are a set of N5W to N5E joints, spaced from 1 foot to over 10

INCORPORATED

12/02/02

Division of Oil, Gas & Mining

Revised: 03/04/93	2-10

feet apart at the surface.  They are only occasionally observed at coalbed depths.  The last two orientations are a system of conjugate shear fractures and joints which are more commonly observed on the surface but occasionally appear at depth and are oriented at approximately N60W and S70W respectively.

As illustrated in Drawings 2.2.1-1 and 2.2.1-2, the majority of the approximately north-south trending faults west of the Connelville die out or terminate in the area of an east to west trending fault in Sections 22, 23, and 24, Township 13 South, Range 6 East.  North of this fault, the majority of the faults and fractures trend approximately east to west.  These faults appear to be sub parallel to the Fish Creek Graben located a few miles north of the North Lease area.  While mining activities were occurring in Mine #3, the in-situ stress were measured in the rocks.  The results of the test indicated the rocks were in compression in an east-west direction (Oral Communication, Mark Bunnell, Skyline Mine Geologist, July, 2002).  Similar measurements were taken in Mine #2, and while less successful, the results of the testing indicated the rocks were in extension in an east- west direction.  Most of the north-south trending faults appear to be normal faults with vertical offset and a slight amount of horizontal offset.  The east-west trending faults can have both vertical and horizontal offsets.  Mines #2 and #3 are separated by the east-west trending fault in Section 22, 23, and 24 (Mines #1, #2, and #3 are illustrated on Drawing 2.2.7-7).  The change in the stresses appears to have a direct impact on the ground water inflow to the mine.  Since the north-south trending faults are in extension, it is theorized that the water held under piezometric pressure within the sandstone units of the Star Point is released at mine faces where fault gouge is weak or gaps are present between fracture faces. The extensional forces would allow for the water to be forced into the space created by “pulling apart” the rocks.  In the area where the rocks are in compression, the east-west trending faults do not have the same “gaps” as formed in the extensional areas and thus the water does not appear to have pathways through the fractures and faults within the Star Point Sandstone.  Drilling records and interviews with well site geologist indicate that in almost all areas of the mine, significant water is not encountered within the Blackhawk Formation.  This suggests that the Blackhawk Formation continues to function as an aquitard whether the rocks are in the state of compression or extension.

Section 2.2.7, Mineable Coal Deposits, is confidential and consequently has been removed and submitted to the Division in a separate folder.

INCORPORATED

12/02/02

Division of Oil, Gas & Mining

Revised: 10/04/02	2-11

Pages 2-12 through 2-15 in Confidential file.

2.2.8  Other Mineral Deposits

Natural gas is the only mineral resource, other than coal, reported in the permit area.  The Clear Creek Gas Field was discovered in 1951 and is apparently nearing depletion.  Two boreholes, presumably natural gas tests, have been drilled and abandoned on the site area.  The T. F. Kerns No. 1 was drilled to a depth of 5,825 feet in Section 13, Township 13 South, Range 6 East.  In Section 23 of the same Township and Range, the Superior Oil Federal No. 1-23 was drilled to 362 feet.  Plate 2.2.8-1 shows the locations of both wells.  Because the Superior Oil well was abandoned and never completed to mining depth or gas depth, full extraction mining will likely occur beneath it.  The T.F. Kerns well will be protected by a 100 foot barrier as shown on Plate 2.2.8-1.

Geochemistry

Analyses were performed on 24 core samples of roof and floor strata for each of the three seams to be mined.  The potential acid-forming or alkalinity-producing materials in the strata to be affected by mining are assessed (See Table 2.2.8-1).  Samples included sandstone, siltstone, and shale material.  Many samples contained carbonaceous material.  No relationship is apparent between any of the analyses and the lithologic or stratigraphic position of a particular sample.  The samples are all slightly alkaline and low in sulfur content.

Clay content of floor samples was not determined analytically.  The lithology of the stratum immediately below the mineable coals varies from borehole to borehole.  Accordingly, clay content will range from almost 100% in a pure claystone to less than 5% in a submature or mature sandstone.

Pyrite, marcasite, and sulfur content of the three mineable coal seams are determined by the standard "forms of sulfur" analysis (Table 2.2.8-2).  Marcasite was not determined directly for the following reasons:

INCORPORATED

12/02/02

Division of Oil, Gas & Mining

2-16

2-17

TABLE 2.2.8-1

SUMMARY OF 24 ANALYSES *

Max.		Min.		Mean		Standard

ANALYSIS								Deviation

Water Soluble Alkalies

(Dry Basis)

%	Na2 O		0.025		0.007		0.009		0.004

%	K2 O		0.032		0.005		0.018		0.010

Sulfur Forms

(Dry Basis)

%  Pyritic		0.83		0.03		0.14		0.18

%  Sulfate		0.03		0.00		0.01		0.01

%  Organic (Diff)	0.42		0.00		0.05		NA

%    Total		1.26		0.02		0.20		0.27

Acidity (pH)

Equipotential		8.58		7.10		8.04		0.37

1:5		8.92		7.27		8.45		0.39

1:20		9.06		7.10		8.65		0.54

*  Individual analyses are available at the mine site.

INCORPORATED

12/02/02

Division of Oil, Gas & Mining

2-18

TABLE 2.2.8-2

U. O'Connor		L. O'Connor B	L. O'Connor A

Sulfur Forms		Percent		Percent		Percent

(Dry Basis)

Pyritic		0.22			0.10			0.19

Sulfate		0.00			0.00			0.00

Organic (Diff.)	0.52			0.43			0.40

TOTAL		0.74			0.53			0.59

INCORPORATED

12/02/02

Division of Oil, Gas & Mining

2-19

●	The standard wet chemical analysis (ASTM D 2492) determines iron soluble in nitric acid, and calculates from this the pyrite content.  This test does not separate marcasite values from pyrite values and merely reports the total as pyrite.

●	On a dry basis the three mineable seams average from 0.55 to 0.62 percent total sulfur by weight.  On the same basis, pyritic sulfur averages from 0.07 to 0.21 percent.  Even if all the pyritic sulfur were marcasite, it would not be sufficient to cause acid mine drainage problems.

●	The Permittee consulted with Commercial Testing and Engineering Laboratories, Inc. in Denver, Colorado, and Standard Laboratories, Inc. in Charleston, West Virginia, and did not find an accurate and quantitative method to determine marcasite content in coal.

The North Lease is a continuation of mining the Lower O’Connor “A” seam as was previously performed in Mine #3 (the existing Mine #3 workings are located within 50 feet horizontally of the planned new works of Mine #3 in the North Lease), the same geochemical conditions are anticipated.  Lab analysis from drill holes in the North Lease of the floor, roof, and coal  indicate the potential for generating acid or toxicity are low.  Results are included in Appendix Volume A-3 Volume 2.   Testing of material transported to the waste rock disposal site indicates that only a very small percentage of material that would be classified as acid and toxic forming has been found in the Lower O’Connor “A” seam.  The handling of waste rock and material determined to be acid or toxic forming is described in Sections 4.4.5 and 4.16 of this M&RP.

2.2.9  Waste Rock Disposal Site

The stratigraphy of the waste rock disposal site area is very similar to that of the minesite permit area, consisting of inter- bedded sandstone, siltstone, and shale, with numerous carbonaceous and coaly zones.  In November of 1976, Sanders Exploration drilled a borehole about 1,300 feet east of the waste rock permit site located in the SE 1/4, NW 1/4, Sec 4, T 13S, R 7E.  The drill hole report for the site identified as S-4 can be found in Appendix Volume A-4.

Geotechnical

The geotechnical data report by Dames and Moore dated October 30, 1979 is included in its entirety in Appendix Volume A-3.  Much of that report is interpretive in nature and deals with facilities that have since been constructed.

INCORPORATED

03/28/08

Division of Oil, Gas & Mining

Revised: 10/25/07	2-20

2.2.10  General Geology of the Rock Disposal Site

The coal-bearing Blackhawk Formation makes up the surface of the rock disposal site.  This formation consists of alternating, laterally discontinuous layers of sandstone, siltstone, shale and coal.  Only occasional sandstone ledges are exposed at the surface of the proposed site, with the remaining surface being covered with up to 20 feet of soil and weathered rock debris.

Two mineable coal seams occur beneath the site, including the Upper and Lower O'Connor seams.  The pertinent data for these coal beds is as follows:

Coal Bed      	Thickness	Depth below Surface

Upper O'Connor	   8.0'	        45'

Lower O'Connor	  18.0'	       130'

Four faults of undetermined displacement have been mapped near the site.  These faults are generally north-south trending and have acted as local barriers to mining in coal mines near the site.

Conversations with Mr. Frank Helsten of Scofield, Utah on September 17, 1981 and May 17, 1982, revealed that the strip mining work was done from 1948 to 1950.  Mr. Helsten was the spot hole driller and indicated that no abandoned underground workings were intercepted when drilling the seam lying 45 feet beneath the floor of the pit.  Mining of the below-lying seam was planned but not accomplished due to economic conditions at the time.

Two drill holes (92-91-03MW and S-4) in the general area of the proposed disposal site provide the basis of the available geological information. Well logs are found in Appendix A-4.  The well locations are shown on Plate 2.3.6-1.  Cross-section A-A’ (Plate 2.2.1-2) shows the geology of the waste rock area.

Revised: 03/04/93	2-21

INCORPORATED

12/02/02

Division of Oil, Gas & Mining

2.2.11 Plans for Casing and Sealing Holes

All exploration drill holes not completed as ground water monitoring wells will be plugged and abandoned using procedures specified by the BLM or the Division.  Typically, exploration holes are backfilled with cement to a point at least thirty feet above the uppermost mineable coal seam.  A bentonite grout is then placed on top of the cement to within 100 feet of the surface.  Surface casings will be removed to at least two feet below ground surface if possible.  The remainder of the hole is  filled to the surface with a neat cement grout.  Occasionally, the governing agency may request a survey monument be placed in the cement cap.

If the exploration hole is to be completed as a monitoring well, it will be constructed by a State licensed driller and in accordance with the requirements set forth by the State Engineer’s Office for monitoring well completions.  Typical well construction will be as follows.  Well screen with appropriately sized apertures and steel casing will be installed in the drill hole to below the lowest mineable coal zone in water-bearing strata.  The screened zone will be sand packed and sealed from overlying strata with at least 2 feet of  bentonite and the overlying hole annulus will be cemented to the  surface.  Well casing with a locking lid will be left at the surface extending above the surface approx. 2 ft.  The wellhead will be properly identified with either a brass marker or a welded-on identification.

INCORPORATED

07/29/10

Division of Oil, Gas & Mining

Once a ground water monitoring well is no longer in use, it will be completely plugged with a cement or cement/bentonite slurry to the to ground surface.  The wellhead and casing will be removed to at least two feet below ground surface when possible.  The surface will be reclaimed to approximate original contour.

In 2009, two (2) drill holes were developed to transfer rock dust from the surface to the underground workings.  Each 3.5-inch hole (3-inch I.D) is approximately 255 feet in length, and completed with steel casing.  At reclamation, the abandonment procedure outlined for exploration holes (at beginning of this section).

2.2.12 Winter Quarters Ventilation Facility

The Winter Quarters Ventilation Facility will be constructed to provide adequate ventilation for mining

Revised: 12-30-09	2-21(a)

north of Winter Quarters Canyon.  The ventilation facility will include a 20-foot diameter vertical shaft, and / or a 20-foot wide slope driven at 18 degrees down, and 8-foot diameter escape shaft.  The 20-foot shaft will have a 12-inch thick concrete liner,  the slope will have a 8-inch thick concrete invert with the ribs and roof having a minimum 3-inch thick shotcrete liner, and the escape shaft will have a 6-inch concrete liner.  When sealing at reclamation, the shaft(s) per 30 CFR Part 75.1711-1 and R645-301-551 will be completely backfilled to the surface using an engineered fill.  When sealing the slope, sealing will consist of solid, substantial, incombustible material, such as concrete blocks, bricks or tile, or shall be completely filled with incombustible material for a distance of at least 25 feet into the opening.  See Section 4.9 for additional details.

2.2.13  Subsidence Monitoring

Please refer to Section 4.17 - Subsidence Control Plan for details of the Subsidence Monitoring program and commitments to mitigate any effects due to subsidence.

Revised: 10-19-12	2-21(b)

INCORPORATED

07/29/10

Division of Oil, Gas & Mining