2.8	 AQUATIC WILDLIFE RESOURCES

Introduction

Both Huntington Creek and Eccles Creek flow through the project area and both provide habitat for reproducing resident populations of cutthroat trout.  Cutthroat trout from Electric Lake use upper Huntington Creek for spawning and nursery activities.  Scofield Reservoir, although stocked with rainbow trout exclusively, has numerous cutthroat trout which have been produced in Eccles Creek and other tributary streams such as Winter Quarters, Woods Creek, Lost Creek, Fish Creek, Pondtown Creek, Pleasant Valley Creek and possibly Boardinghouse Creek.

Eccles Creek

Eccles Creek is a small mountain stream draining west to east into Pleasant Valley Creek which flows north approximately 3 miles where it empties into Scofield Reservoir.  Discharges in Eccles Creek are frequently as low as 2 cfs during late summer, fall and winter months; and high flows seldom exceed 50 cfs, even at the creek mouth.  Water temperatures of streams such as Eccles Creek fluctuate because of turbulence from the rough channels.  During November to March, water temperatures remain between 0-20 C.  In the summer, water temperatures often fluctuate from 12-150 C daily although high temperatures seldom exceed 200 C.

Through natural erosion of mudstone, sandstone and shale deposits, Eccles Creek has periods of high total suspended solids (sedimentation).  This occurs, however, during periods of high runoff when the stream waters have sufficient energy (velocity) to carry the fine sediments out of the canyon rather than depositing them on the coarser substrate materials.  During normal runoff years, there are numerous clean trout spawning gravel beds in Eccles Creek.  During low runoff years, fine sediments may not be flushed from the spawning grounds.  Maintenance of this resource is dependent upon a continuation of flow, adequate substrate, food base and water quality conditions.

The existing aquatic species of Eccles Creek, fish and macroinvertebrates, have adapted to tolerate natural temperature fluctuations and sediment loads.  The macroinvertebrate communities of Eccles Creek have a high diversity of species representing all major trophic groups.  There are

Revised: 08-24-05				2-65

INCORPORATED

12/02/05

Division of Oil, Gas & Mining

species found only in high quality water streams indicating the high water quality of Eccles Creek.  There are also environmentally resistant taxa present.  This high diversity represents a resiliency to environmental change, especially short term changes.

Upper Eccles Creek above the  White Oak Mine Road (at sampling stations ECO3, ECO2, UPMF, UPSF, Figure 2.8-A), have numerous taxa of macroinvertebrates found only in high quality waters and stable habitats.  Lower Eccles Creek (Stations ECO4 and ECO5) has a more tolerant macroinvertebrate community with taxa tolerant to sedimentation dominating the community.

Cutthroat trout have historically maintained naturally reproducing populations in Eccles Creek from the National Forest boundary downstream to the creek mouth.  Recent migrations, however, have been impeded by obstacles unrelated to mining, such as, irrigation diversions, and numerous beaver dams.  There are no fish in the upper forks of Eccles Creek.

Construction activities caused sufficient sediment loading into Eccles Creek resulting in a significant reduction in fish populations.  Cooperative efforts with DWR, however, resulted in habitat improvement by 1986 sufficient for near recovery of these populations.  (See DWR report, "Recovery of the Cutthroat Trout (Salmo Clarki) Fishery in Eccles Creek, Utah From Coal Mining Impacts", Donaldson & Dalton, Volume A-3, under Aquatic Wildlife.)

INCORPORATED

12/02/05

Division of Oil, Gas & Mining

Huntington Creek

Huntington Creek above Electric Lake is a small mountain stream draining north to south into Electric Lake.  Discharges are frequently as low as 1.5 cfs, with spring high flows sometimes reaching 100 cfs.  Water temperatures fluctuate during summer months, although daily highs seldom exceed 20NC.  During winter months water temperatures seldom exceed 2NC and the stream is nearly completely iced over.

Through natural erosion of existing stream channel geological formations and adjoining hillsides, Huntington Creek waters have high loads of fine sediments during runoff periods.  This occurs when runoff is high and these sediments are carried into Electric Lake rather than being deposited on spawning gravels or other coarse substrate materials.  Huntington Creek has numerous clean trout spawning gravel beds and the amount of fine sediments is not high enough to hinder spawning.

Revised: 08-24-05				2-66

Huntington Creek has a diverse aquatic community with macroinvertebrate taxa representing all trophic levels.  The successful cutthroat trout spawning and high number of resident trout evidence the high quality waters and habitat of Huntington Creek plus the ability of the macroinvertebrate community to support quality fisheries.  Cutthroat trout, according to Utah Division of Wildlife Resources (UDWR) surveys, are increasing in numbers in Huntington Creek above Electric Lake.  Trout produced in Huntington Creek provide an important part of the total number of fish in Electric Lake.

Winter Quarters Canyon Creek

As indicated in the 1995 environmental assessment prepared by the Forest Service and the Bureau of Land Management Winters Quarters Canyon Creek has a moderate population of macroinvertebrates.  Perennial flow in the canyon has produced Stonefly larvae as far up as Box and Bob=s Canyons.  Mayfly nymphs were also found present in waters tested.  Cutthroat trout were found within the creek east of the Forest Boundary on June 7, 1994 indicating fish are likely within perennial sections of the creek containing significant flows.   A survey conducted in Winter Quarters Canyon Creek in October 2002 indicated similar conditions and species (See Appendix Volume A-3, Volume 2).  The Winter Quarters Ventilation Facility pad was specifically designed to minimized any potential impacts to the stream.  The pad was designed to stay a minimum of two(2) stream widths from the stream, ( or approximately 24 feet), thus maintaining a buffer zone and avoiding impacts to both the stream and riparian areas.  The macroinvertebrates are monitored on a scheduled basis to insure the health of the stream (see Plate 2.8.1-1 for locations, Table 2.8-1a for monitoring frequency).  Refer to Section2.4.3 - sediment yield and next section for measures implemented to construct in the stream buffer zone.

Woods Canyon Creek

As indicated in the 1995 Environmental Assessment, Mayfly nymphs were found within the upper portions of Woods Canyon Creek in higher quantities than those found within Winter Quarters Canyon.  Stonefly larvae were also found as high as the fork in the stream near the center of Section 34 (T 12 S, R 6 E).  No fish were seen during the 1994 field survey although some may have been present.  A survey conducted in Woods Canyon Creek in October 2002 indicated similar conditions (See Appendix Volume A-3, Volume 2).  Another fish survey was conducted in 2010 to serve as baseline information for expanded mining located approximately ½ mile east and further downstream than previously conducted.  Similar with the earlier surveys the stream is relatively shallow and does not provided ideal fish habitat, however a total of eight (8) fish were identified.  An addition of both a macroinvertebrate and a fish monitoring location were set up to insure monitoring stations are established downstream of mining activities to fully evaluate any impacts from mining.  Details are outlined later in this section.

Fish Creek

The North Lease area was extended in 2012 to include approximately 770 acres in the Fish Creek drainage.  No perennial streams are being undermined.  Perennial flow in Wife Canyon begins approximately 0.35 miles downstream of the area being undermined.  The West Fork of Andrew Dairy Canyon is perennial only in short reaches, and the East Fork of Andrew Dairy Canyon is ephemeral. Approximately 900-1300 feet of overburden exist in the area being undermined, further minimizing any impacts.  Andrew Dairy Spring, which exists immediately outside the area to be mined is being monitored as Spring S25-32.  Water Right 91-3917 is a Spring located above the area to be mined and will be monitored S26-1 .  No monitoring of aquatic resources is necessary in these drainages.

UP Canyon - Scofield Waste Rock site

The Scofield Waste Rock site is located in UP Canyon at the confluence of two ephemeral unnamed drainages.  No aquatic wildlife habitat has been noted in either drainage.

Revised: 10-19-12									2-67

INCORPORATED

02/09/11

Division of Oil, Gas & Mining

Project Impacts on Fisheries Resources

The surface facility disturbances in the portal area encroached on sections of all three upper Eccles Creek forks.  In order to reduce sedimentation of these stream segments and the main stream, the tributaries and a section of Eccles Creek proper immediately below the tributary confluences were diverted into closed culverts.  This modified approximately 4,200 feet of total stream habitat but did not reduce available fish habitat since fish were not found above the U.S. Forest boundary, prior to the diversion.  Downstream drift of macroinvertebrates from the upper reaches of these forks still occurs as before.

At the coal loadout facilities near the mouth of the canyon (Station ECO5), approximately 600 feet of stream was moved to the north into a new channel.  The new channel is 100 feet shorter but has nearly the same gradient (3 feet additional vertical drop/1,000 feet horizontal channel).

Degradation of Eccles Creek between the National Forest boundary and the coal loadout facilities should continue to be minimal since road and conveyor plans were developed and are being implemented to minimize effects on the stream.

Water being discharged from the mine is augmenting the Eccles Creek stream flow.  This increased stream flow is especially beneficial during summer months when normal stream flows are low.  Water temperatures are also moderated by this increased flow.

There should be little impact on Huntington Creek above Electric Lake.  Impacts to date have been associated only with the construction of a new UDOT highway.  Sediment control measures minimized the impact during the construction activity.

Prior to construction of the Winter Quarters Ventilation Facility (WQVF) silt fencing or similar best management practice will be installed along the entire length of the construction zone to minimize sediment and debris from entering the creek. Once construction is complete and other sediment controls are installed, these situation structures will be removed.  During the life of the WQVF pad, long term sediment control will be implemented thorough a sediment pond (UPDES discharge point 004).

At this point in time there are believed to be no other potential impacts on either Winter Quarters or Woods Canyon Creeks.

INCORPORATED

07/29/10

Division of Oil, Gas & Mining

Revised: 3-24-10				2-68

figure 2.8A

2.8.1  Aquatic Monitoring Program

An aquatic monitoring program has been conducted to meet the intent of the requirements of R614-301-311.  The main purpose of the monitoring program is to gain sufficient knowledge to prevent and/or minimize impacts through wise project planning.  Monitoring has: (1) described existing resources; (2) detected existing perturbations; and (3) provided the basis for wise project planning, operation and resource restoration.

The biological (macroinvertebrate and fish) and habitat (sediment and channel surveys) monitoring stations on Eccles Creek are shown in Figure 2.8-A.  Sampling dates are limited by weather, but June and late October samplings are usually possible.  Two seasonal sampling dates per year are required to differentiate natural seasonal intrastand variance from impact induced changes.  Samples were taken annually through the project planning and early development.  The biological sampling has been performed in conjunction with normal stream water monitoring so that comparative analysis is possible.

Seven stations on Eccles Creek were selected in relation to impact areas, UDWR fish sampling stations, existing macroinvertebrate and sediment stations, and water quality monitoring stations.  At each station on the scheduled sample date (Table 2.8-1), four macroinvertebrate samples were taken from selected optimal substrates with a modified Surber Sampler.  Three sediment samples were taken from potential spawning grounds.  Replicate samples were taken to enable an analysis of variance between samples.

Habitat surveys, following methodologies used by USBLM and USDFS fisheries habitat specialists, were made annually throughout construction at critical Eccles Creek stations (Table 2.8.1).  Measurements included:  stream bank stability, channel substrate composition; stream gradient; riparian vegetation (type, relative cover); water width, depth and velocity at various discharges (Q) and channel width and tortuosity.

Fish surveys are conducted by UDWR personnel out of the Price office.  Fish surveys are usually made in August so year class I fish will be large enough to sample and young-of-year fish are large

enough to observe.  Fish are measured as to total length and weight, counted and then released.  These data are compared with earlier UDWR collection records, thus illustrating present fish population conditions compared with years past.

Table 2.8-2 summarizes the stream monitoring data.  A summary of the sediment composition data, taken in accordance with the schedule on Table 2.8-1, is shown on Table 2.8-3.  The UDWR reports have been added to the Aquatic Wildlife section of Volume A-3.

Revised: 08-24-05				2-70

INCORPORATED

12/02/05

Division of Oil, Gas & Mining

James and Burnout Creeks

A fish and macroinvertebrate monitoring program for James and Burnout Creeks will be implemented prior to undermining the lower portion of James Canyon.  The fish survey consists of multi-pass electro fishing to estimate fish populations in the streams for one year and then every third year thereafter.  The fish surveys will be performed in the fall of each third year on or about the same date, and will continue through a minimum of two (2) years following active subsidence (2008).  The fish survey was initiated on October 16, 2000.

A macroinvertebrate survey of Burnout Creek will be performed twice a year for two consecutive years and then every three years thereafter. This survey will be performed in the fall and spring of each year, with the fall macroinvertebrate survey will being performed in conjunction with the fall fish survey.  The macroinvertebrate survey was initiated on October 16, 2000.

A macroinvertebrate survey of James Creek initiated on October 16, 2000 will be performed twice a year for two consecutive years and then every three years thereafter or for a period determined by Canyon Fuel Company, LLC, DOGM, USFS and the DWR, to be long enough to provide data to establish population trends . This survey will be performed in the fall and spring of each year on or about the same date.  The fall macroinvertebrate survey will be performed in conjunction with the fall fish survey. (See Table 2.8-1a for Sampling Scheduled).

The following methods have been used previously on this creek and will continue to be used for future macroinvertebrate sampling.  Slight variations to the methods may occur during the field work or based on comments from regulatory agencies.

Three benthic sites will be sampled.  They will be located beginning approximately 162 meters (528 feet) above the reservoir.  Refer to Plate 2.8.1-1 for all macroinvertebrate sample site locations.  Note that macroinvertebrate sites sample reaches of a stream - site locations are plotted at the approximate mid-point of the reach of the sample site.

Quantitative samples will be taken with a modified box sampler.  The three samples taken will be field preserved in 70% ethyl alcohol and returned to the laboratory for processing.   The samples will be sorted and invertebrates identified to the lowest possible taxonomic level using the keys of Merritt and Cummins (1996).  Those of questionable identity will be further examined and identified under

INCORPORATED

02/01/11

Division of Oil, Gas & Mining

Revised: 09-5-07				2-71

magnification.  The mean, standard deviation, density per square meter, and standing crop will be calculated and estimated using the same methods as in previous analysis.

Calculations of the USFS Biotic Condition Index (Winget and Mangum 1979) will be completed using the abundances of the benthic taxa to generate the dominance weighted community tolerant quotient (CTQd).  The predicted community tolerant quotient (CTQp) will be calculated using water chemistry data provided in Winget (1972) for the Huntington Creek drainage.

Cluster analysis will be run using the Bray-Curtis dissimilarity index with the UPGM clustering algorithm.

Winter Quarters Canyon and Woods Canyon Creeks

From Fall of 2002 through early Summer of 2004 fish and baseline macroinvertebrate data for the perennial reaches within Winter Quarters Canyon and Woods Canyon Creeks  in the North Lease area were gathered.  Copies of the reports are included in Appendix Volume A-3, Volume 2.

A macroinvertebrate survey of portions of Winter Quarters Canyon and Woods Canyon Creeks will be performed twice a year for two consecutive years and then every third year thereafter or for a period determined by Canyon Fuel Company, LLC, DOGM, USFS, and the DWR, to be long enough to provide data to establish population trends. This survey will be performed in the fall and spring of each year on or about the same date.

In 2010 the Winter Quarters Ventilation Facility (WQVF) was added to the permit area approximately 2 mile downstream of the existing macroinvertebrate monitoring stations.  Consultation with Dr. Shiozawa who directs the Skyline marcoinvertebrate monitoring program, indicated the portion of stream in the vicinity of the WQVF pad is not conducive to a macroinvertebrate study due to low gradient and inundation of fine sediment.  He recommended a electro-fishing monitoring program which is outlined later in this section.

As mining progressed north of Winter Quarters Canyon, the longwall panel orientation was rotated 90 degrees to maximize coal recovery.  The rotation expanded mining approximately ½ mile to the east.  To accommodate the modification, an additional macroinvertebrate station and fish monitoring station were set up in Woods Canyon to insure monitoring stations are extablished downstream of mining activities to fully evaluate any impacts from mining.  The additional electro-fishing monitoring station was added to Woods Canyon creek in 2010 although the stream is marginal fish habitat due to the shallow nature.  Sampling frequency will continue every 3rd year unless future sampling confirms the habitat is unsuitable to sustain a viable fish population.  See Appendix Volume A-3, Volume 2 for 2010 fish density report.

The following methods have been and will be used for macroinvertebrate sampling.  Slight variations to the methods may occur during the field work or based on comments from regulatory agencies.

Three benthic sites will be sampled in each creek.  Following the first survey a map with these stations will be prepared and submitted with the next sample report (included in the following year=s annual report).  Quantitative samples will be taken with a modified box sampler.  The samples taken will be field preserved in 70% ethyl alcohol and returned to the laboratory for processing.   The samples will be sorted and invertebrates identified to the lowest possible taxonomic level using the keys of Merritt and Cummins (1996).  Those of questionable identity will be further examined and identified under magnification.  The mean, standard deviation, density per square meter, and

Revised: 12-29-10			2-71a

INCORPORATED

02/09/11

Division of Oil, Gas & Mining

standing crop will be calculated and estimated.

Calculations of the USFS Biotic Condition Index (Winget and Mangum 1979) will be completed using the abundances of the benthic taxa to generate the dominance weighted community tolerant quotient (CTQd).  The predicted community tolerant quotient (CTQp) will be calculated using water chemistry data provided in Winget (1972) for the Huntington Creek drainage.

Cluster analysis will be run using the Bray-Curtis dissimilarity index with the UPGM clustering algorithm.

An electro fishing study  was conducted in 2002 to examine 1) the species present in Winter Quarters Canyon; 2) determine if fish were present in Woods Canyon; and 3) determine how far upstream fish extended into either canyon.  The one-time survey was conducted on request by the U.S. Forest Service (See Appendix A-3, Volume 2 for report).

Based on the addition of the Winter Quarters Ventilation Facility, beginning in 2010 two (2) electro fishing sites were established Winter Quarters Creek.  Two sampling runs (150 meters in length), one upstream and one downstream of the WQVF pad, will be tested on an tri-year basis to monitor the general aquatic health of Winter Quarters Creek.  Sampling is minimized to every third year to reduce the stress on the fish population.  Emergency sampling could be implemented on an as-needed basis.

The above described  macroinvertebrate studies will continue for two years after active mining and subsidence stops in the applicable drainages unless statistical analysis demonstrates impacts have occurred or may occur to these streams. In the event subsidence causes quantifiable damages to fish populations, stream flows, or other negative impacts on fish or wildlife habitat, the mine will identify, research and implement measures sufficient to correct the problems.  If monitoring data suggests potential adverse impacts have occurred but cannot be conclusively demonstrated by the three years of collected data, a new monitoring plan to determine the type and extent of the impact will be developed by DOGM, DWR, USFS and Skyline Mine.  The additional monitoring will continue until such impacts are either demonstrated or dismissed by joint consent of DOGM, DWR, USFS, and Skyline Mine.  Areas where there is potential for habitat loss from subsidence are shown on Plate 4.17.3-1a.  The consumption rate of water from mining activities is provided in Section 2.5.2.

Future aquatic monitoring is planned only on an as needed basis.  Need will be established in conjunction with UDWR personnel and will be required only in case of a major perturbation in fish populations or other anomalous conditions.  Monitoring data will be reviewed for mining related

impacts, and, if found, a mitigation plan will be developed in conjunction with UDWR and UDOGM personnel.  The Permittee will cooperate with UDWR in the investigation of any such conditions.  This approach to future monitoring is consistent with the requirements recommended by the UDWR, Price office.

Revised: 3-24-10					2-71b

INCORPORATED

07/29/10

Division of Oil, Gas & Mining

Eccles Creek

To determine the impacts to Eccles Creek , if any, related to discharge of mine water, semi-annual macroinvertebrate studies will be conducted.  The studies will start in the spring of 2002 and will continue for three (3) consecutive years or for a period determined by Canyon Fuel Company, LLC, DOGM and the DWR, to be long enough to provide data to establish population trends.

The following methods have been used previously on this creek and will be used for future sampling.  Slight variations to the methods may occur during the field work or based on comments from regulatory agencies.

Four samples will be taken at intervals separated by approximately 20 to 30 m.  Samples will continue to be taken from three stations on the stream, with five replicates per station.  These stations on Eccles Creek are located above South Fork (site 1), Eccles Creek at Whisky Canyon (site 2), and Lower Eccles Creek (site 3).   A map with these stations has been added to the Aquatic Wildlife section of Volume A-3.

A box sampler will be used to collect the samples.  Samples will be taken in areas with rubble or cobble substrates to insure that similar habitats are examined.  When possible, samples will be taken from parts of the stream channel that had been submerged continuously throughout the year.  The substrate will be stirred to a depth of approximately 5 cm.  Rocks within the area of the sampler will be removed and washed to insure quantitative assessment of the invertebrates.  The box sampler will have a net mesh of approximately 250 microns.  The samples will be concentrated on a screen with a mesh of approximately 64 microns and field preserved in ethyl alcohol.

In the laboratory the samples will be sorted in a pan illuminated from below.  All invertebrates will be removed and identified to the lowest possible taxonomic level using the keys of Merritt and Cummins (1996).  The mean density and standard deviation per sample will be calculated for each taxon and the mean values will be used to determine the density per square meter.

The collected data will be compared to the previous data collected in 1979 through 1983 and included in Appendix A-3, Volume 2.  After comparing historical data with data obtained in the three year study starting in 2002, a determination will be made if the macroinvertebrate in Eccles Creek are impacted by the mine water discharge.  If impacts are found, the significance of the impacts will be determined and appropriate mitigation is necessary will be performed.

Fish studies will be conducted on Eccles Creek in 2003.The fish study consists of multi-pass electro fishing to estimate fish populations in the stream for one year and then every third year

08-24-05				2-71c

INCORPORATED

11/14/07

Division of Oil, Gas & Mining

thereafter.  The fish studies will be performed in the fall of each year.  Results from sampling will be compared with all available historic studies done on this creek.

Aquatic Monitoring Summary

Aquatic monitoring data have been accumulated for a period of six years.  Summaries of these data are presented in Tables 2.8.2 and 2.8.3.  Backup data for these summaries, including the consultants original report and subsequent summary reports, may be found in Appendix Volume A-3, Aquatic Wildlife.

During a 23 year time span macro-invertebrate data was collect on 24 site visits to Eccles Creek and compiled into a report entitled AA Compilation and Comparison of Eccles Creek Macro-Invertebrate Data for the Period of 1979 - 2002" prepared by Dennis Shiozawa, Ph.D., Professor of Fisheries Biology at Brigham Young University, Provo, Utah.  The report documents that Eccles Creek has undergone changes in the benthic community structure through the years of study and demonstrates the resilience and adaptability of the creek to disturbance by multiple sources.  Burnout, James, Eccles, Winter Quarters, and Woods canyon creeks have continued Aquatic monitoring reports that are submitted to the Division of Oil, Gas, and Mining in the required Annual Report when the information is compiled.  These reports are available for review at the Division of Oil, Gas and Mining public library.

INCORPORATED

12/02/05

Division of Oil, Gas & Mining

Revised 08-24-05				2-71d

TABLE 2.8-1

STREAM RESOURCE MONITORING SCHEDULE FOR ECCLES CREEK

STATION

Survey			June			August		October

______________________________________________________________________________

Macroinvertebrates	Eccles Creek

UPSF, UPMF, EC02,

ECO3, ECSF, ECO4,

ECO5				None			Same as June

______________________________________________________________________________

Fish			None			Eccles Creek			None

ECSF, EC03,

EC04, EC05

______________________________________________________________________________

Sediments		Eccles Creek						Eccles Creek

EC02, EC03,				 		EC02, EC03,

EC04, EC05		None				EC04, EC05

______________________________________________________________________________

Habitat		            Eccles Creek

EC02, EC03,

EC04, EC05		None				None

______________________________________________________________________________

2-72

INCORPORATED

Division of Oil, Gas & Mining

| Table 2.8-1a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2000 |  | 2001 |  | 2002 |  | 2003 |  | 2004 |  | 2005 |  | 2006 |  | 2007 |  | 2008 |  | 2009 |  | 2010 |  | 2011 |  |
| Sample Site | End Date | Spring | Fall | Spring | Fall | Spring | Fall | Spring | Fall | Spring | Fall | Spring | Fall | Spring | Fall | Spring | Fall | Spring | Fall | Spring | Fall | Spring | Fall | Spring | Fall |
| Fish |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Burnout | F, 2007 |  | C |  | C |  |  |  |  |  | C |  |  |  |  |  | C |  |  |  |  |  |  |  |  |
| Eccles | ND |  |  |  | C |  |  |  |  |  | C |  |  |  |  |  | C |  |  |  |  |  | X |  |  |
| James | F, 2007 |  | C |  | C |  |  |  |  |  | C |  |  |  |  |  | C |  |  |  |  |  |  |  |  |
| Winter |  |  |  |  |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |
| Woods | RC |  |  |  |  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |
|  |  | 2012 |  | 2013 |  | 2014 |  | 2015 |  | 2016 |  | 2017 |  | 2018 |  | 2019 |  | 2020 |  | 2021 |  | 2022 |  | 2023 |  |
| Eccles | ND |  |  |  | X |  |  |  |  |  | X |  |  |  |  |  | X |  |  |  |  |  | X |  |  |
| Winter |  |  |  |  | X |  |  |  |  |  | X |  |  |  |  |  | X |  |  |  |  |  | X |  |  |
| Woods | RC |  |  |  | X |  |  |  |  |  | X |  |  |  |  |  | X |  |  |  |  |  | X |  |  |
| Macroinvertebrate |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2000 |  | 2001 |  | 2002 |  | 2003 |  | 2004 |  | 2005 |  | 2006 |  | 2007 |  | 2008 |  | 2009 |  | 2010 |  | 2011 |  |
| Burnout |  |  | C | C | C | C | C | C | C | C |  |  |  |  |  |  | RC | RC |  |  |  |  |  |  |  |
| Eccles | ND |  |  |  | C | C | C | C | C | C | C |  |  |  |  |  | C | C |  |  |  |  |  | X | X |
| James |  |  | C | C | C | C | C | C | C | C |  |  |  |  |  |  | RC | RC |  |  |  |  |  |  |  |
| Winter | 2yr ptm |  |  |  |  |  | C | C | C | C |  |  |  |  |  |  | C | C |  |  |  |  |  | X | X |
| Woods | 2yr ptm |  |  |  |  |  | C | C | C | C |  |  |  |  |  |  | C | C |  |  |  |  |  | X | X |
|  |  | 2012 |  | 2013 |  | 2014 |  | 2015 |  | 2016 |  | 2017 |  | 2018 |  | 2019 |  | 2020 |  | 2021 |  | 2022 |  | 2023 |  |
| Eccles | ND |  |  |  |  |  |  |  | X | X |  |  |  |  |  |  | X | X |  |  |  |  |  |  | X |
| Winter | 2yr ptm |  |  |  |  |  |  |  | X | X |  |  |  |  |  |  | X | X |  |  |  |  |  |  | X |
| Woods | 2yr ptm |  |  |  |  |  |  |  | X | X |  |  |  |  |  |  | X | X |  |  |  |  |  |  | X |
| Key: C = completed, X = scheduled, ND = no end date, F = Fall, RC = requirements completed, ptm = post mining, |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Reports located in the Annual Submitted to the Division of Oil, Gas, and Mining. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Revised: 1-25-11 |  |  |  |  |  |  |  |  |  |  |  | 2-72a |  | INCORPORATED 02/01/11 Division of Oil, Gas & Mining |  |  |  |  |  |  |  |  |  |  |  |

TABLE 2.8-2

DESCRIPTORS OF THE BENTHIC COMMUNITIES AT SEVEN STATIONS ON

ECCLES CREEK AND ITS MAJOR TRIBUTARIES, CARBON COUNTY, UTAH

#			              Standard

Date		CTQa	CTQd	taxa	  H      	    #/m2   	  Deviation	 gm/m2

UPSF - Upper South Fork Eccles Creek above mine portal and culvert

Jun 79		69	72	30	3.800       10,176	     9,072             --

Jun 80		74	77	22	3.715         7,728	     2,470	  1.39

Aug 80	            62	64	33	3.453         2,892	        868            0.81

Nov 80	            71	71	31	3.855       14,834	     6,898	  4.45

Jun 81		67	69	23	3.619       10,545	     4,437            1.31

Sep 81	            64	63	32	3.453       31,185	   20,570	  3.19

Nov 81     	66	66	32	3.197       36,788	     2,358	  6.67

Jun 82		68	70	29	3.850         4,608	     1,961	  1.58

Aug 82  	68	71	33	3.406       24,457	     9,715	  8.87

Oct 82		70	70	33	2.634	    34,868	   21,970	  2.50

Oct 83		66	66	34	3.788	    29,477	   13,557	  3.70

Jun 84		67	67	35	3.399	    13,821	     4,543            2.70

Sep 84	            71	72	31	2.908	    24,842	   14,305	  6.80

Oct 84		68	66	33	2.523	    42,666	   23,968	  3.90

Jun 85		66	69	32	3.173	    28,778	     6,224            4.30

Jul 85		64	65	28	3.573	    13,378	     7,997	  5.30

Oct 85		55	55	28	2.873	    27,438	     2,180	  1.70

UPMF - Upper Middle Fork Eccles Creek above mine portal and culvert

Jun 79		67	68	30	3.150	     8,449	     2,650	   --

Jun 80		72	74	26	3.884	     4,019	     2,029	   0.62

Aug 80      	63	63	33	3.114	    17,090	   14,450	   1.59

Nov 80  	70	64	38	3.844	    44,127	   25,316	   1.44

Jun 81		65	68	35	3.301	    41,684	     9,427             4.15

Sep 81	            70	70	34	3.542	    50,134	   47,174	   4.79

Nov 81	            61	64	32	3.148	    65,792	   13,774	   3.66

Jun 82		68	69	30	3.862	      8,113	     3,814	   1.87

Aug 82	            65	65	33	3.199	    33,332	   11,605	 12.60

Oct 82		59	59	38	3.427	    55,457	   10,794	 10.78

Oct 83		61	64	27	1.852	    46,497	   10,006	   6.10

Jun 84		66	68	24	1.848	    13,316	     2,325             1.00

Sep 84   	59	63	33	2.959	    26,338	   13,028	   5.10

Oct 84		59	60	28	2.118	    54,411	   12,092	 10.20

Jun 85		62	66	29	3.043	    13,991	     4,100             3.30

Jul 85		57	58	29	3.391	    11,858	     6,664	   3.90

Oct 85		56	53	29	2.467	    41,512	     4,204	   5.20

2-73

INCORPORATED

Division of Oil, Gas & Mining

Table 2.8-2 (Continued)

#			              Standard

Date		CTQa	CTQd	taxa	      H   	        #/m2 	  Deviation	gm/m2

EC-02 - Eccles Creek immediately below mine portal and mouth of

culvert

May 79   	66	66	36	  3.510	       12,339	    5,138	    --

Aug 79	            65	65	42	  1.964	       73,181	  22,640	 3.26

Oct 79		64	66	32	  2.534	       17,761	  11,601	 2.61

Jun 80		64	63	27	  3.389	         4,350	    1,874	 1.57

Aug 80    	68	69	31	  3.468	         5,232	    1,253	 0.44

Nov 80	            66	69	30	  3.020	         9,745	    2,414	 0.54

Jun 81		64	63	21	  2.173	       11,274	    4,645            2.20

Sep 81	            71	73	30	  2.554	       19,077	  19,191	 1.09

Nov 81  	80	84	24	  3.970	         2,370	    1,418	 0.51

Jun 82		72	71	30	  2.411	         6,053	    4,898	 1.18

Aug 82	            64	69	26	  2.514	         7,798	    4,659	 1.75

Oct 82		70	72	30	 1.838	       28,718	  16,615	 6.37

Oct 83		70	74	23	 3.746	         2,556	       917	 3.30

Jun 84		67	71	21	 1.040	       14,843	    6,779            2.80

Sep 84	            70	73	29	 2.923	         4,070	       719	 1.70

Oct 84		66	69	28	 2.142	       11,190	    2,263	 3.80

Jun 85		71	74	22	 2.849	         5,544	    1,845	 1.70

Jul 85		65	67	24	 2.770	       13,758	    5,891	 3.90

Oct 85		65	69	23	 2.037	       13,391	    1,040	 1.60

EC-SF - South Fork Eccles Creek below EC-02 and above EC-03

May 79 	59	60	36	 3.510	         9,321	   7,243	 --

Aug 79  	64	66	35	 3.322	       17,773	 10,151            1.53

Oct 79		68	65	37	 3.289	       10,453	   4,180	1.51

Aug 80   	69	68	38	 3.134	         6,994	   4,681	1.87

Oct 80		61 	69	33	 2.634	       17,243	 17,178	0.73

May 81   	66	69	29	 3.408	         3,532	   1,539	0.47

Sep 81	            62	65	40	 2.681	       39,070	 26,699            7.68

Jun 82		64	62	24	 2.939	         6,136	   2,394	2.58

Aug 82	            61	63	29	 2.811	       20,460	   8,987	6.59

Oct 82		63	64	33	 2.572	       38,228	 15,785          11.83

Oct 83		64	64	36	 2.607	       14,276	 15,120            3.70

Jun 84		62	61	29	 2.483	       13,278	   5,901            2.10

Sep 84  	59	59	38	 2.808	       27,739	  9,145	          11.20

Oct 84		58	59	32	 1.900	       71,992      35,479          10.50

Jun 85		66	65	35	 2.699	       15,852	  3,160             5.30

Jul 85		58	57	30	 2.942	       17,567	 11,776           10.80

Oct 85		52	53	29	 2.445         34,540	   3,327	 5.10

INCORPORATED

Division of Oil, Gas & Mining

2-74

Table 2.8-2 (Continued)

#			                            Standard

Date		CTQa	CTQd	taxa	      H   	       #/m2             Deviation	           gm/m2

EC-03 - Eccles Creek below South Fork and above Whiskey Springs

May 79  	65	62	27	  2.450	      18,093	      8,455	        --

Aug 79	            55	44	30	  2.743	      23,247	    10,395	       3.07

Oct 79		63	61	34	  2.892	      15,871	    11,841	       1.40

Apr 80		70	65	23	  2.407	      26,251	      5,119	       2.03

Aug 80	            70	64	23	  2.472	        6,873	      4,914	       0.73

Oct 80		57	57	34	  1.904       58,069	    31,108	       3.95

May 81	            73	74	23	  3.201	        3,882	      3,136                0.58

Sep 81    	75	79	24	  1.383	      13,585	      4,717	       1.01

Jun 82		80	81	15	  2.688	           619	         384	       0.18

Aug 82   	71	73	26	  2.424	      11,193	      4,389	       7.56

Oct 82		69	72	29	  2.107	      16,008	    16,189	       2.62

Oct 83		68	65	28	  3.567	        4,931	      1,120	       2.90

Jun 84		66	69	29	  2.106	      15,790	      9,154                1.60

Sep 84	            63	63	25	  3.086	        2,749	         911	       2.10

Oct 84		57	61	25	  2.228	      10,615	      5,188	       1.50

Jun 85		59	60	21	  2.155	        7,231	      2,697	       3.50

Jul 85		59	60	26	  1.869	      16,789	      7,024	       2.20

Oct 85		60	62	22	  1.619	      25,781	      1,655	       0.80

EC-04 - Eccles Creek below Whiskey Springs and Belina Road

May 79	            63	62	35	  2.450	      11,634	       7,222	          --

Aug 79   	61	62	37	  3.060	      25,273	     10,619	       2.29

Oct 79		60	62	39	  2.227	      34,233	     22,843	       2.16

Apr 80		61	61	28	  2.301	      13,420	     11,040	       2.25

Aug 80	            70	67	29	  2.676	        5,130	       2,319	       1.32

Oct 80		62	65	37	  0.973	      46,338	     15,816	       1.20

May 81	            74	74	23	  0.973	        6,607	       2,162	       1.76

Sep 81   	72	75	29	  1.291	      31,347	     12,003	       1.87

Jun 82		83	86	14	  2.292	           683	          436	       0.62

Aug 82	            67	71	21	  1.927	      13,520	       5,937	       2.40

Oct 82		66	71	26	  1.678	      11,207	       5,382	       1.37

Oct 83		60	63	28	  1.645	      18,908	      10,402	       4.30

Jun 84		67	72	18	  0.579	      13,969	        4,259              1.80

Sep 84  	59	69	27	  3.002	        5,159	        1,104	       4.80

Oct 84		56	57	30	  2.689	      11,621	        1,828	       5.60

Jun 85		74	74	23	  2.008	        4,939	           715	       5.00

Jul 85		69	69	25	  2.366	        5,204	        1,505	       4.20

Oct 85		57	61	27	  2.037	        4,288	           572	       0.50

INCORPORATED

Division of Oil, Gas & Mining

2-75

Table 2.8-2 (Continued)

#			                        Standard

Date		CTQa	CTQd	taxa	      H   	         #/m2 	Deviation	gm/m2

EC-05 - Eccles Creek at mouth of canyon (quantitative habitat

study site)

May 79  	59	59	28	  2.280	     18,661	12,773	              --

Aug 79	            74	75	21	  2.590	       2,526	  1,066	 	0.55

Oct 79		65	70	32	  2.155	     14,308	  6,806	            3.49

Apr 80		73	76	28	  2.319      12,560	  3,907	            3.70

Aug 80  	64	71	24	  2.057	       6,085	  1,851	 	1.76

Oct 80		59	61	33	  1.865	     34,303        16,125	            1.42

May 81  	58	64	26	  1.441	       9,870	  2,460	 	2.44

Sep 81	            59	65	27	  2.544	     15,909	  6,457	 	3.91

Jun 82		79	84	14	  1.863	       1,216	     332	 	0.49

Aug 82	            70	76	16	  1.363	     17,609	  7,710	 	3.11

Oct 82		70	73	24	  0.990	     22,631	  5,258	            6.12

Oct 83		75	79	19	  2.160	       6,047	  2,547	 	0.80

Jun 84		75	81	16	  1.042	       5,431	  2,730	            0.90

Sep 84	            71	72	23	  0.828	     41,232	  5,591	 	2.60

Oct 84		67	67	21	  0.615	    51,680         19,698	            4.20

Jun 85		70	73	20	  1.256	    11,419	  2,383             5.50

Jul 85		67	74	17	  1.661	    19,691         12,370	            3.10

Oct 85		53	64	22	  1.511	    18,001	  1,437	            1.30

INCORPORATED

Division of Oil, Gas & Mining

2-76

TABLE 2.8-3

SEDIMENT COMPOSITION OF GRAVEL BEDS

AT FOUR STATIONS ON ECCLES CREEK

AS PERCENT OF TOTAL MEAN WEIGHT

FOR SEDIMENTS PASSING THROUGH SIX USGS

STANDARD SOIL SIEVE SIZES

Sieve

Opening	Aug	Aug	Oct	May	Oct	Oct

(in mm)	1979	1980	1980	1981	1982	1983

Station ECC-02

12.7		44.5	51.0	---	59.8	59.8	55.7

4.75		26.7	34.2	---	39.5	48.8	36.8

2.00		17.9	25.7	---	28.5	32.6	27.2

2.00		17.9	25.7	---	28.5	32.6	27.2

0.85		13.6	21.3	---	20.0	26.6	21.8

0.50		12.2	18.4	---	15.3	22.4	17.6

0.074	 	0.2	 1.8	---	 1.9	 2.6	 0.3

Station ECC-03

12.7		---	55.2	58.1	73.2	65.9	59.2

4.75		---	38.8	45.8	53.9	40.9	44.8

2.00		---	29.5	39.5	40.9	31.8	37.5

0.85		---	25.3	35.1	35.0	28.8	32.5

0.50		---	22.7	29.5	31.8	26.0	23.8

0.074		---	 1.8	 0.4	 1.9	 1.6	 0.7

Station ECC-04

12.7		45.2	48.3	48.1	32.1	49.5	79.7

4.75		26.2	30.5	29.4	18.9	26.6	57.2

2.00		17.5	22.8	20.6	10.4	17.6	43.6

0.85		13.5	18.9	17.2	7.1	13.5	33.5

0.50		12.2	15.5	14.5	4.4	11.2	23.5

0.074		 0.2	 1.8	 0.2	 0.9	 2.1	 0.8

Station ECC-05

12.7		50.4	43.9	41.7	38.9	46.2	53.3

4.75		29.4	28.1	21.8	19.5	33.9	31.2

2.00		20.3	22.1	16.7	13.2	28.6	20.2

0.85		15.9	18.9	14.3	10.4	24.5	14.3

0.50		14.1	14.9	12.2	 7.8	20.7	 9.1

0.074	 	 0.2	 1.4	 0.2	 1.0	 2.0	 0.3

INCORPORATED

Division of Oil, Gas & Mining

2-77

TABLE 2.8-3

SEDIMENT COMPOSITION OF GRAVEL BEDS

AT FOUR STATIONS ON ECCLES CREEK

AS PERCENT OF TOTAL MEAN WEIGHT

FOR SEDIMENTS PASSING THROUGH SIX USGS

STANDARD SOIL SIEVE SIZES

Sieve

Opening	June	Oct	June	Oct

(in mm)	1984	1984	1985	1985

Station ECC-02

12.7		66.5	64.2	54.4	69.5

4.75		41.6	40.8	31.8	50.6

2.00		29.2	30.6	20.5	22.6

0.85		23.0	25.8	15.0	14.4

0.50		18.8	22.6	11.8	11.1

0.074	 	  0.8	 3.4	 0.5	 0.9

Station ECC-03

12.7		67.2	58.1	75.7	74.0

4.75		47.8	39.6	47.0	51.3

2.00		41.7	29.6	33.2	28.5

0.85		39.3	24.1	26.0	18.1

0.50		36.9	20.4	21.1	13.3

0.074	 	1.7	 2.6	 1.2	 2.0

Station ECC-04

12.7		63.6	56.5	69.1	73.5

4.75		39.7	34.0	37.5	55.5

2.00		27.8	22.6	25.6	31.7

0.85		20.4	17.0	21.1	19.3

0.50		16.4	13.9	17.0	14.7

0.074	 	1.1	 1.1	 1.1	 1.2

Station ECC-05

12.7		57.4	55.6	62.1	62.5

4.75		35.4	32.0	32.3	39.5

2.00		25.5	22.7	21.6	32.9

0.85		21.2	18.4	17.5	23.9

0.50		18.8	15.1	14.4	19.6

0.074	 	1.4	 1.5	 1.1	 1.6

INCORPORATED

Division of Oil, Gas & Mining

2-78

References

Merritt, R. W. and K. W. Cummins.  (eds.)    1996.  An Introduction to the

Aquatic Insects of North America.  Kendall/Hunt Publishing Co.  Dubuque, Iowa. 862pp.

Winget, R. N. and F. A. Mangum.  1979.  Biotic condition index: integrated

biological, physical, and chemical stream parameters for management. U.S. Forest Service Intermountain Region.  Ogden, UT.

Winget, R. N.  1972.  Aquatic environmental impact study of Huntington 	Canyon

generating station and Electric Lake.  Annual Report No. 2.  Center for Environmental Studies.  Brigham Young University.

INCORPORATED

12/02/02

Division of Oil, Gas & Mining

Revised 10/04/02                                                            2-78a